ORCHESTRA: A Simulation-Based Human-Centred Multi-Agent Framework for Predictive Maintenance Decision Support in Laser Welding

Abstract—Predictive maintenance in laser welding must translate condition observations into a timely intervention while jointly considering weld quality, production continuity, resource constraints, and operator trust. This paper presents ORCHESTRA—Optimised Real-Time Coordination for Human-Centred Edge Scheduling, Task Response and Analytics—a simulation-based, four-agent decision-support framework. A Machine/IoT Agent represents laser-welding condition signals; an AI Predictive Agent estimates maintenance urgency with XGBoost; a Human Operator Agent is selectively invoked under uncertainty, conflicting evidence, or operational sensitivity; and a PPO-based Scheduling Agent selects among five deferral/maintenance actions. The study reframes the proof-of-concept around laser-welding-relevant variables, including laser energy/power, focus position, assist-gas conditions, optical/plasma signatures, melt-pool/thermal signals, and positioning/mechanical indicators. It retains only controlled prior numerical results as explicitly labelled simulation proxies. Full ORCHESTRA achieves planning performance comparable to PPO-only scheduling while routing a material subset of cases through expert review. The central contribution is the auditable coordination architecture, not a new XGBoost or PPO algorithm. No industrial deployment or live laser dataset is claimed.

Keywords-Predictive Maintenance; Laser Welding; Human-In-The-Loop; Multi-Agent Systems; XGBoost; PPO; Maintenance Scheduling; Condition Monitoring

# 1. Introduction

Laser welding is increasingly important in automotive, battery, and precision fabrication owing to its high power density, narrow heat-affected zone, and compatibility with automation [1]. Its maintenance problem is not a generic equipment-fault task: a serviceable decision depends on coupled process and equipment conditions, including beam delivery, focus position, assist gas, plume/plasma behaviour, melt-pool stability, thermal evolution, and part/robot positioning. Degradation need not cause immediate stoppage; a shifted focus or contaminated optical path may first appear through changes in reflected light, plume stability, or weld consistency. Maintenance is therefore a constrained sequential decision: intervene too late and quality/reliability risk rises; intervene too often and availability and cost suffer.

A prediction-only alert is insufficient because it does not account for production load, technician availability, spares, the cost of stopping a cell, or contextual cues available to an operator. Purely autonomous reinforcement-learning (RL) scheduling can adapt to such variables, but may be difficult to govern when predictive confidence is poor or process signals conflict. Human-in-the-loop (HITL) systems address this gap, yet commonly leave the operator outside the scheduling loop. Multi-agent manufacturing research offers a natural organisational abstraction, but often separates prediction, human judgement, and scheduling [2].

This paper proposes ORCHESTRA as a decision-support architecture that integrates these functions while retaining a transparent boundary between observed signals, derived condition indicators, and simulated maintenance outcomes. Laser welding is the primary case. Resistance/spot welding is retained only as the source of a prior adapted simulation dataset and controlled baseline logic; it is not presented as an experimentally matched comparison.

Contributions. (1) A human-centred, four-agent framework for laser-welding maintenance decision support. (2) Integration of XGBoost maintenance-urgency prediction with uncertainty-triggered operator review. (3) A context-aware PPO scheduler that considers severity, uncertainty, production load, resource availability, time since last maintenance, and cost. (4) A transparent simulation evaluation with baselines, ablations, and sensitivity analyses. The study is a simulation-based proof-of-concept, not a full industrial deployment.

# 2. Related Work

Predictive maintenance (PdM) in smart manufacturing is evolving from condition acquisition and degradation prediction toward prescriptive analytics. Reviews identify data quality, label scarcity, and actionability as persistent barriers [3]. In a manufacturing cell, prediction is only one input to a resource-allocation and scheduling decision.

Laser-welding monitoring and maintenance. In-situ monitoring studies provide a defensible sensing basis. High-speed CCD imaging has been used to visualise melt-pool dynamics, while spectrometers capture laser-induced plasma emission; plasma-spectrum features have been related to weld-bead characteristics and defects [1]. Other work reports high-speed imaging and spectroscopy for galvanized-steel lap welding, linking plume/keyhole behaviour, zinc vaporisation, and defects [4]. These studies establish observable process signals and fault-sensitive mechanisms, but do not provide an integrated maintenance dataset containing failure labels, maintenance actions, operator interventions, and production costs.

Machine learning for maintenance prediction. Tree ensembles such as XGBoost are attractive for tabular sensor-derived features because they accommodate nonlinearities and permit relatively low-latency deployment. Our use of XGBoost is an implementation choice; no new learner is proposed. Feature importance can support selective review, but must not be confused with physical causation.

RL maintenance scheduling. RL is suited to sequential maintenance under stochastic demand and resource constraints. Multi-agent deep-RL PdM on parallel machines has shown substantial improvement over corrective and fixed preventive policies in its setting [2]. That result is not transferred to the present reward or laser process; the architectural lesson—that prediction and scheduling should be coupled—is adopted.

HITL and multi-agent manufacturing. HITL is valuable when confidence is low, labels are noisy, or production impact is high. Agent-based manufacturing supports distributed roles for assets, controllers, operators, and planners [5]. Nevertheless, prediction, selective human escalation, and PPO scheduling are rarely connected in one auditable loop.

| Integration theme | Existing strength | Remaining limitation | ORCHESTRA response |
| --- | --- | --- | --- |
| ML PdM | Accurate degradation/quality inference | Often stops at an alert | Converts urgency into scheduled actions |
| RL maintenance | Adapts timing to state and cost | Can exclude human context | Uses uncertainty-triggered review |
| HITL systems | Improves trust and exception handling | Often advisory only | Human output enters PPO logic |
| Multi-agent manufacturing | Distributed coordination | Prediction, review, scheduling fragmented | One framework with explicit feedback |

Table 1: Research-gap and novelty positioning.

Novelty positioning. ORCHESTRA’s novelty lies in the coordination architecture and its evidence labels: the Machine/IoT Agent exposes provenance; the Human Operator Agent is selectively triggered rather than universally required; and the PPO scheduler incorporates the resulting contextual action. The paper does not propose a new XGBoost or PPO algorithm.

# 3. ORCHESTRA Framework

ORCHESTRA receives a time-indexed process/condition vector, predicts a continuous maintenance-urgency score, decides whether to escalate to an operator, and selects a maintenance action. It is a decision-support system, not a closed-loop safety controller. The architecture is modular: implementations may substitute sensors, predictors, or planners without changing the information contract.

Figure 1: Proposed ORCHESTRA architecture. Machine/IoT Agent → AI Predictive Agent → (optional) Human Operator Agent → PPO Scheduling Agent → maintenance system, with outcome feedback. [Author task: replace placeholder with vector graphic.]

| Agent | Main role | Inputs | Outputs |
| --- | --- | --- | --- |
| Machine/IoT | Condition acquisition and normalisation | Laser process/sensor records, timestamps, equipment state | Validated feature vector; provenance tags |
| AI Predictive | Maintenance-urgency estimation | Direct/derived features | Urgency score, severity, uncertainty |
| Human Operator | Review of selected cases | Prediction, uncertainty, explanations, production context | Validation, refinement, or override |
| RL Scheduling | Context-aware action selection | Severity, uncertainty, load, resources, time, cost, human note | Action 0–4 and rationale |

Table 2: Agent roles, inputs, and outputs.

Machine/IoT Agent. This agent abstracts the edge interface; it does not infer maintenance. Candidate laser-welding inputs include laser power/energy, pulse/duty settings, beam/wavelength metadata, focus position, assist-gas flow/purity proxies, co-axial/off-axis optical emission, plasma/plume indicators, melt-pool/keyhole vision, infrared/thermal signatures, reflectivity proxies, acoustic/vibration signals, and robot/CNC positioning residuals. A specific cell measures a subset dictated by optics, joint geometry, material, and sensor access. Setpoint deviations and operating context are retained because parameter changes are not necessarily degradation.

AI Predictive Agent. XGBoost maps the curated feature vector to a continuous urgency score u ∈ [0,100]. Severity is a post-processed band: low (u<33), medium (33 ≤ u<66), and high (u ≥ 66). Uncertainty is represented through a transparent proxy—predicted variance/interval width, ensemble disagreement, or an out-of-distribution score—not an uncalibrated softmax confidence.

Human Operator Agent. The operator is explicitly simulated in this proof-of-concept. A review object is created when severity is medium, uncertainty is high, indicators conflict, or a high-impact action coincides with constrained resources. The review can validate, refine, or override; the scheduler logs the reason and result. This limits both unsupported autonomy claims and review burden.

| Condition | Review? | Output / routing |
| --- | --- | --- |
| Low severity, low uncertainty | No | Direct to PPO scheduling |
| Medium severity | Yes | Validate or refine |
| High uncertainty | Yes | Contextual review |
| High severity, high confidence | Optional | Urgent action may proceed directly |
| Conflicting indicators / resource conflict | Yes | Override or scheduling refinement |

Table 3: Uncertainty-triggered human-review logic.

RL Scheduling Agent and feedback. The PPO agent maps a maintenance state to one of five actions. Its contribution is context-aware action formulation, not an algorithmic change to PPO.

| Action | Maintenance decision |
| --- | --- |
| 0 | Continue monitoring / no immediate maintenance |
| 1 | Immediate maintenance |
| 2 | Schedule in the next low-load window |
| 3 | Schedule in three days |
| 4 | Schedule in seven days |

Table 4: PPO action space.

Communication follows a fixed contract: a condition message flows to prediction; a prediction message flows to review/scheduling; a review message modifies the scheduling context; a scheduling message records timing and resource effects; and a feedback message returns outcomes. This makes routing auditable and supports future offline policy improvement.

# 4. Methodology

We adopt a simulation-based design because the exact integrated target—public laser-welding signals together with maintenance events, human decisions, resource states, and production costs—was not available as a verified dataset. The pipeline is: define agent contracts; construct a reproducible laser-inspired state representation; train/evaluate XGBoost; simulate uncertainty-triggered review; train PPO in a Gymnasium-like environment; and compare baselines, ablations, and sensitivities.

Laser-welding case and variable provenance. The simulation uses a physically motivated state, but values are generated under stated assumptions. Directly monitored variables are those that a sensor could record; derived variables are calculated from them; simulated variables are generated because maintenance labels or outcomes are absent.

| Variable | Role | Status / source type |
| --- | --- | --- |
| Laser power/energy; pulse metrics | Input | Directly observed (simulated stream) |
| Focus position / focal offset | Input | Directly observed (simulated stream) |
| Assist-gas flow, pressure/purity proxy | Input | Directly observed (simulated stream) |
| Optical/plasma emission / spectral feature | Input | Direct/derived |
| Melt-pool/keyhole image feature | Input | Derived from simulated vision stream |
| Thermal/infrared signature | Input | Direct/derived proxy |
| Reflectivity / back-reflected-light proxy | Input | Derived proxy |
| Vibration/acoustic and positioning residual | Input | Direct/derived proxy |
| Drift/health state | Latent | Simulated |
| Maintenance urgency score | Target | Derived/simulated |
| Time since last maintenance, load, resources, cost | Context state | Simulated |

Table 5: Variable provenance for the laser-inspired simulation.

Data-source statement. We do not claim that a single public dataset supplies all columns above. If an open laser-welding multimodal corpus becomes available, it should replace the synthetic streams. A controlled resistance/spot-welding comparison is omitted from the main results because electrode-current, electrode-force, and ultrasonic variables are not directly transferable to laser process physics.

XGBoost model. Features are normalised after median-based imputation and clipping. The target is a continuous maintenance-urgency score. Several interpretable/ensemble learners were compared for model-selection purposes; XGBoost was selected for tabular efficiency and ease of feature inspection, not because universal superiority is claimed. An 80/20 split and five-fold cross-validation are used; random seeds and hyperparameter ranges are recorded.

| Element | Setting / rationale |
| --- | --- |
| Objective | reg:squarederror for continuous urgency |
| Max depth | 6 |
| Learning rate | 0.05 |
| Estimators / subsample / colsample | 500 / 0.8 / 0.8 |
| Evaluation | MAE, MSE/RMSE, R², calibration/out-of-range checks |
| Interpretability | Gain-based importance plus per-case explanation |

Table 6: XGBoost configuration.

The headline model-selection comparison is retained as illustrative proxy values: simple-MLP mean CV accuracy 93.69%, CV time 26.91 s, MAE 1.92; XGBoost Fast 93.62%, 5.07 s, 1.68. These are not evidence of laser-welding prediction accuracy and must be recomputed for the reframed simulation.

Human review mechanism. Let h denote human availability and c operational conflict. A review flag r = 1 when any of the following holds: medium severity; uncertainty q ≥ τq; conflicting signal directions; or a high-impact action under resource conflict. Otherwise r = 0. The simulated operator samples a rule-constrained response that validates, refines, or overrides based on a latent “true” simulated condition. This is a transparent model of human behaviour, not an empirical measurement of operators.

PPO scheduling environment. The state is s_t = [S_risk, Q, L, R, T, C, H], where S_risk is predicted severity/urgency, Q uncertainty, L normalised production load, R resource availability, T time since last maintenance, C maintenance-cost factor, and H human-review status/refinement. The action is discrete {0,1,2,3,4}. An episode terminates after a fixed horizon or a critical failure. PPO uses a clipped objective, GAE λ = 0.95, and separate policy/value networks; exact network sizes, rollout length, and seed are implementation parameters.

The reward balances safety and operations: r_t = w_safe r_safe + w_risk r_risk + w_down r_down + w_cost r_cost + w_prod r_prod. A critical failure incurs a large negative term; timely intervention under high risk is rewarded; deferring under high risk is penalised; unnecessary immediate maintenance under low risk is penalised; and scheduling during low-load windows is preferred when risk is moderate. Reward weights are scenario assumptions, not calibrated plant economics.

Baselines and evaluation. Baselines are reactive (maintain at failure), fixed preventive, XGBoost-only alert, rule-based scheduling, PPO-only, and Full ORCHESTRA. Ablations remove human review, PPO scheduling, or both. Metrics are mean episode reward, critical-failure count, preventive-action share, downtime proxy, cost proxy, human-review rate, and override rate. Results are averaged over common random seeds where available. Because all maintenance outcomes are simulated, differences demonstrate internal consistency rather than field efficacy.

# 5. Results and Discussion

This section reports only results that can be transparently retained. The underlying numbers are carried over from the prior spot-welding proof-of-concept as illustrative laser-welding proxy values; they must be recomputed on laser-specific data before supporting substantive empirical claims.

Prediction and feature importance. XGBoost produced a continuous urgency estimate calibrated for the simulated labels. A regression example retained from prior runs reported MSE 6.24, MAE 1.68, and R² 0.9953; these numbers are not evidence for a real laser process. Feature importance should prioritise variables with defensible causal roles: focus drift and optical/plasma features can affect energy coupling and weld stability; thermal and melt-pool features reflect process state; vibration/positioning features capture mechanical disturbance. Correlated inputs can split importance arbitrarily, so grouped or SHAP analysis is recommended in a physical implementation.

Figure 2: Feature-importance template. Generate from the reframed laser simulation; retain exact feature names from Table 5. [Placeholder]

Planning value of scheduling. The retained proxy result distinguishes alerting from scheduling. XGBoost-only alert (273.46) and XGBoost plus human review (267.36) score well below PPO-only (613.90) and Full ORCHESTRA (608.60). The interpretation is that action selection and timing dominate the measured proxy objective, not that XGBoost is ineffective. Full ORCHESTRA is nearly identical in reward to PPO-only in this controlled setup while retaining a route for human escalation.

| Strategy (proxy) | Mean reward | Downtime proxy | Cost proxy | Review % |
| --- | --- | --- | --- | --- |
| XGBoost-only alert | 273.46 | 6.97 | 11.36 | 0.00 |
| XGBoost + Human | 267.36 | 6.88 | 11.20 | 38.42 |
| Rule-based | 434.49 | 6.02 | 25.90 | 0.00 |
| PPO-only | 613.90 | 6.66 | 28.07 | 0.00 |
| Full ORCHESTRA | 608.60 | 6.55 | 27.93 | 38.66 |

Table 7: Baseline/ablation proxy comparison. Values are illustrative and not measured laser-welding results.

Selective review and sensitivity. In the proxy experiment, increasing the uncertainty threshold reduced review and override rates. This is a useful design finding only under the simulation’s reward and review model: a plant should tune τq to its risk tolerance, technician workload, and uncertainty calibration. A second noise sensitivity retained from the prior study showed reward 608.60 at zero noise and 645.46 at 0.20 noise amplitude; downtime rose from 6.55 to 9.02 and cost proxy from 27.93 to 30.11. These values demonstrate a testable mechanism, not robustness of an installed system.

| Threshold (proxy) | Mean reward | Review % | Override % |
| --- | --- | --- | --- |
| 0.30 | 523.27 | 86.08 | 15.08 |
| 0.50 | 582.59 | 53.94 | 9.40 |
| 0.60 | 608.60 | 38.66 | 7.12 |
| 0.70 | 622.92 | 27.46 | 5.02 |
| 0.90 | 632.56 | 21.62 | 4.00 |

Table 8: Uncertainty-threshold sensitivity (illustrative proxy).

Why the full framework can be comparable rather than superior. PPO-only already receives urgency and operational state; human review affects only a minority of states in the simulation. If review corrects a small share of harmful decisions, aggregate reward may change little. The framework’s practical value therefore lies in governed exception handling, traceability, and operator acceptance—metrics that the current proxy does not adequately quantify.

# 6. Conclusion

ORCHESTRA formulates laser-welding maintenance as coordinated sensing, prediction, selective human review, and context-aware scheduling. Its four contributions are balanced: a human-centred multi-agent architecture; XGBoost urgency prediction with uncertainty-triggered escalation; PPO scheduling with operational context; and a reproducible simulation protocol with baselines, ablations, and sensitivity. The laser-specific sensing rationale is grounded in melt-pool imaging, plasma/plume spectroscopy, thermal behaviour, and process stability [1], while observed, derived, and simulated quantities are clearly separated.

Limitations. No public integrated laser-welding dataset with the exact maintenance/action/operator/cost labels was identified; hence the reported numeric outcomes are controlled prior proxy values. Human review is modelled rather than empirically measured with operators. Reward weights, cost, failure dynamics, resource constraints, and action delays are assumptions. Resistance/spot-welding is not used for a quantitative cross-process claim. The action space and PPO settings require plant-specific calibration.

Future work. The immediate priority is a laser-cell digital twin or partnered-cell study that records optical/thermal/vision signals, maintenance events, technician decisions, and production impact. This would replace proxy labels, support calibrated uncertainty, and enable evaluation of operator workload and trust alongside cost/downtime. Deployment should follow safety, cybersecurity, and governance review.

# References

[1] S. H. M. et al., “Real-time monitoring of the laser hot-wire welding process,” Journal of Materials Processing Technology, 2013. [verify DOI/author before submission].

[2] M. L. Ruiz Rodríguez et al., “Multi-agent deep reinforcement learning based predictive maintenance on parallel machines,” Robotics and Computer-Integrated Manufacturing, vol. 78, p. 102406, 2022. https://doi.org/10.1016/j.rcim.2022.102406.

[3] M. Compare, P. Baraldi, and E. Zio, “Challenges to IoT-enabled predictive maintenance for Industry 4.0,” IEEE Internet of Things Journal, vol. 7, no. 5, pp. 4585–4597, 2020. https://doi.org/10.1109/JIOT.2019.2957029.

[4] “Real-time monitoring of laser welding of galvanized high-strength steel in lap-joint configuration,” SMU Research Publication. [accessed: 2026-09-02].

[5] T. Pulikottil et al., “Agent-based manufacturing—review and expert evaluation,” International Journal of Advanced Manufacturing Technology, vol. 127, pp. 2151–2180, 2023. https://doi.org/10.1007/s00170-023-11517-8.

Submission note: Reference [1] requires authoritative publisher/author/DOI confirmation. Reference [4] should be replaced with a primary publication record if available. All numerical tables carried from the prior proof-of-concept must be recomputed for the laser-specific simulation before camera-ready submission.