I am currently preparing a 6–8 page conference paper based on our current manuscript titled:

“ORCHESTRA: A Simulation-Based Human-Centred Multi-Agent Framework for Predictive Maintenance Decision Support in Resistance Spot Welding.”

The current draft is much longer than a standard conference paper, so the main task is to condense, organise, verify, and strengthen the manuscript while keeping the main contribution clear. The paper should focus on the ORCHESTRA framework, which integrates:

- Machine/IoT Agent for resistance spot welding condition data

- AI Predictive Agent using XGBoost for maintenance urgency prediction

- Human Operator Agent for uncertainty-triggered review

- RL Scheduling Agent using PPO for proactive maintenance scheduling

The main contribution is not the development of a new XGBoost or PPO algorithm. The contribution is the integration of prediction, human review, and reinforcement learning scheduling into a human-centred multi-agent decision-support framework.

Please follow the instructions below carefully.

# 1. Target paper structure

Please convert the current draft into the following 6–8 page conference paper structure:

| Section | Target length | Purpose |
| --- | --- | --- |
| Title, Abstract, Keywords | 0.5 page | Present the study clearly and concisely |
| 1. Introduction | 1 page | Problem, research gap, aim and contributions |
| 2. Related Work | 1 page | Concise review and novelty positioning |
| 3. ORCHESTRA Framework | 1.5 pages | Explain the four-agent architecture |
| 4. Methodology | 1.5 pages | Dataset, XGBoost, human review, PPO and evaluation design |
| 5. Results and Discussion | 2 pages | Present only the strongest results |
| 6. Conclusion | 0.5 page | Main findings, limitations and future work |
| References | As required | Conference format |

Please avoid making the paper read like a thesis chapter. It should be concise, focused and suitable for a conference proceeding.

# 2. Introduction

Please reduce the Introduction to approximately one page.

The Introduction should include:

- A short explanation of why predictive maintenance is important in smart manufacturing.

- Why resistance spot welding is a relevant case study.

- The limitation of prediction-only predictive maintenance systems.

- The need to connect prediction with human judgement and maintenance scheduling.

- A clear statement of the ORCHESTRA framework.

- Three or four clear contributions.

Please use this contribution structure:

The main contributions of this paper are:

- It proposes ORCHESTRA, a human-centred multi-agent framework for predictive maintenance decision support in resistance spot welding.

- It integrates XGBoost-based maintenance urgency prediction with uncertainty-triggered human review.

- It develops a PPO-based maintenance scheduling layer that considers severity, uncertainty, production load, resource availability, time since last maintenance and cost.

- It evaluates the framework through a simulation-based proof-of-concept using baseline, ablation and sensitivity analysis.

Please make sure the Introduction clearly states that this is a simulation-based proof-of-concept, not a full industrial deployment.

# 3. Related Work

The current Related Work section needs to be strengthened and condensed. Please rewrite it as a concise synthesis, not a long literature review.

Please organise it around these themes:

- Predictive maintenance in smart manufacturing

- Predictive maintenance and monitoring in resistance spot welding

- Machine learning for maintenance prediction

- Reinforcement learning for maintenance scheduling

- Human-in-the-loop decision support

- Multi-agent systems in smart manufacturing

At the end of the Related Work section, please include a short novelty table like this:

| Literature area | Existing strength | Remaining limitation | ORCHESTRA response |
| --- | --- | --- | --- |
| ML predictive maintenance | Good at fault or urgency prediction | Often weak in action selection | Adds PPO-based scheduling |
| RL maintenance scheduling | Learns adaptive maintenance decisions | Often lacks human contextual input | Adds uncertainty-triggered human review |
| Human-in-the-loop systems | Improves trust and contextual judgement | Often not formally connected to scheduling | Links human review to PPO scheduling |
| Multi-agent manufacturing | Supports distributed decision-making | Often separates prediction, human input and scheduling | Integrates all components in one framework |

Please make sure the novelty is defended clearly. The key research gap is that existing work often treats prediction, human judgement and scheduling as separate tasks, while ORCHESTRA connects them within a single coordinated architecture.

# 4. ORCHESTRA Framework section

Please keep this section focused on the framework architecture.

It should include:

- A short overview of ORCHESTRA.

- A figure showing the framework architecture.

- A concise explanation of the four agents.

- A table summarising the role, input and output of each agent.

- A short explanation of the communication and feedback logic.

Please do not include too much repeated explanation. The reader only needs to understand how the system works.

Please keep or recreate a table similar to this:

| Agent | Main role | Input | Output |
| --- | --- | --- | --- |
| Machine/IoT Agent | Represents the welding process and condition data | Sensor/process variables | Pre-processed condition data |
| AI Predictive Agent | Estimates maintenance urgency using XGBoost | Condition data | Urgency score, severity and uncertainty |
| Human Operator Agent | Reviews uncertain or sensitive cases | Prediction, uncertainty and context | Validation, refinement or override |
| RL Scheduling Agent | Selects maintenance action using PPO | Severity, uncertainty, resources, load and cost | Maintenance scheduling decision |

Please include the key maintenance actions used by the PPO agent:

| Action | Maintenance decision |
| --- | --- |
| 0 | No immediate action |
| 1 | Immediate maintenance |
| 2 | Maintenance during next low-load window |
| 3 | Maintenance in three days |
| 4 | Maintenance in seven days |

# 5. Methodology

Please condense the Methodology section to approximately 1.5 pages.

It should include only the essential information:

## 5.1 Dataset

Please verify the dataset details carefully.

The current draft says the dataset is:

“Resistance Spot Welding Insights: A Dataset Integrating Process Parameters, Infrared, and Surface Imaging”

Please check:

- Correct dataset title

- Correct authors

- Repository source

- DOI or URL if available

- Which variables were directly available

- Which variables were derived, adapted or simulated

This is important because the current study uses some variables that may not come directly from the original dataset. Please make this transparent.

Please include a table like this:

| Variable | Role | Source type |
| --- | --- | --- |
| Electrode force | Input feature | Original/adapted |
| Welding current | Input feature | Original/adapted |
| Ultrasonic inspection score | Input feature | Original/adapted |
| Electrode tip temperature | Input feature | Original/adapted |
| Robot encoder error | Input feature | Derived/simulated |
| Vibration | Input feature | Derived/simulated |
| Predictive maintenance score | Target | Derived/simulated |

Please check the exact classification of each variable. Do not assume that all variables are directly available in the original dataset.

## 5.2 XGBoost model

Please briefly explain:

- Why several models were compared.

- Why XGBoost was selected.

- The input variables.

- The target variable.

- The main hyperparameters.

- The main evaluation metrics.

Do not include long explanations of XGBoost theory.

## 5.3 Human review mechanism

Please briefly explain the uncertainty-triggered review logic:

| Condition | Human review? |
| --- | --- |
| Low severity and low uncertainty | No |
| Medium severity | Yes |
| High uncertainty | Yes |
| High severity and high confidence | Optional/no |
| Conflicting indicators | Yes |
| High severity and low resource availability | Yes |

Please state clearly that the Human Operator Agent is simulated in this version.

## 5.4 PPO scheduling environment

Please include:

- State space

- Action space

- Reward logic

- Training configuration

- Evaluation design

Keep this concise. The paper does not need a long explanation of PPO.

## 5.5 Baselines and evaluation

Please include the baseline strategies:

- Reactive maintenance

- Fixed preventive maintenance

- XGBoost-only alert

- XGBoost + human review

- Rule-based scheduling

- PPO-only scheduling

- Full ORCHESTRA

Please include the main evaluation metrics:

- Mean reward

- Critical failures

- Unnecessary maintenance

- Downtime proxy

- Cost proxy

- Human intervention rate

# 6. Results and Discussion

Please combine Results and Discussion into one section to save space.

The results section should focus only on the strongest evidence. Please include:

## 6.1 Model comparison

Keep the table short. Include only the top five models, not all 18.

Suggested table:

| Model | Accuracy | MAE | CV time |
| --- | --- | --- | --- |
| MLP Simple Network | 93.69% | 1.92 | 26.91 s |
| XGBoost Fast | 93.62% | 1.68 | 5.07 s |
| Gradient Boosting | 93.61% | 1.85 | 282.65 s |
| Random Forest | 93.58% | 2.01 | 29.00 s |
| LightGBM Fast | 93.54% | 1.78 | 7.44 s |

The discussion should say that XGBoost was selected because it provided the best balance between performance, error and computational efficiency.

## 6.2 XGBoost prediction and feature importance

Please keep the XGBoost regression results:

| Metric | Value |
| --- | --- |
| MSE | 6.24 |
| MAE | 1.68 |
| R² | 0.9953 |

Please include a feature importance figure. The discussion should say that electrode tip temperature and vibration were the most influential predictors, which is physically meaningful for resistance spot welding maintenance.

## 6.3 PPO scheduling performance

Please include the main PPO results:

| Metric | Value |
| --- | --- |
| Mean episode reward | 614.39 |
| Standard deviation | 27.87 |
| Critical failures | 0.00 |
| Unnecessary maintenance | 0.30 |
| Downtime proxy | 6.67 |
| Cost proxy | 28.08 |

Please briefly discuss that the PPO agent learned a planning-oriented policy and avoided critical failures under the simulation conditions.

## 6.4 Baseline and ablation comparison

Please include the baseline comparison because this is one of the strongest parts of the paper.

Suggested condensed table:

| Strategy | Mean reward | Critical failures | Downtime proxy | Cost proxy | Human review |
| --- | --- | --- | --- | --- | --- |
| Reactive | -86.92 | 0.00 | 0.00 | 0.00 | 0.00% |
| Fixed preventive | -118.96 | 0.00 | 2.13 | 2.98 | 0.00% |
| XGBoost-only | 273.46 | 0.00 | 6.97 | 11.36 | 0.00% |
| Rule-based | 434.49 | 0.00 | 6.02 | 25.90 | 0.00% |
| PPO-only | 613.90 | 0.00 | 6.66 | 28.07 | 0.00% |
| Full ORCHESTRA | 608.60 | 0.00 | 6.55 | 27.93 | 38.66% |

Please be careful with the interpretation. Do not say that Full ORCHESTRA numerically outperformed PPO-only. The correct interpretation is:

Full ORCHESTRA achieved comparable performance to PPO-only scheduling while adding uncertainty-triggered human review and slightly reducing downtime and cost proxies.

## 6.5 Sensitivity analysis

Please reduce the sensitivity analysis to one short paragraph and, if space allows, one small summary table.

The main point is:

ORCHESTRA remained stable across changes in uncertainty threshold, sensor noise, reward weighting and human intervention frequency, with no critical failures observed under the tested simulation conditions.

Do not include too many sensitivity tables unless there is enough space.

# 7. Figures to prepare

Please prepare only four main figures for the conference paper:

| Figure | Description |
| --- | --- |
| Figure 1 | ORCHESTRA framework architecture |
| Figure 2 | Methodology pipeline |
| Figure 3 | Model comparison: accuracy/error vs computational time |
| Figure 4 | Baseline or ablation comparison of mean reward |

Please make the figures clean, readable and suitable for a conference paper. Avoid small text and overly complicated diagrams.

# 8. References

Please complete the references properly.

The paper needs approximately 15–25 strong references. Please prioritise recent papers from 2020–2026, but include classic references when necessary.

Please find references for:

- Predictive maintenance in smart manufacturing

- Resistance spot welding monitoring and degradation

- XGBoost or machine learning for predictive maintenance

- Reinforcement learning for maintenance scheduling

- Human-in-the-loop decision support

- Multi-agent systems in manufacturing

- The resistance spot welding dataset

Please remove weak or irrelevant references. Please also make sure every citation in the text appears in the reference list and every reference is cited in the paper.

# 9. Claim control

Please carefully check the wording to avoid overclaiming.

Use safer wording like this:

| Avoid | Use instead |
| --- | --- |
| “The framework is validated in industry” | “The framework is evaluated through a simulation-based proof-of-concept” |
| “ORCHESTRA improves industrial maintenance” | “ORCHESTRA demonstrates potential for maintenance decision support” |
| “The human operator improves performance” | “The simulated human review mechanism provides selective contextual refinement” |
| “The system prevents failures” | “No critical failures occurred under the tested simulation conditions” |
| “This model is ready for deployment” | “Future work should validate the framework using live industrial data and real operators” |

This is very important because reviewers may criticise the paper if it claims more than the simulation can support.

# 10. Final writing style

Please use clear academic English and keep the writing concise. The paper should be written in a formal conference style.

Please follow these writing rules:

- Avoid repetition.

- Keep each paragraph focused on one idea.

- Use short and clear sentences where possible.

- Do not over-explain XGBoost or PPO theory.

- Emphasise the framework contribution.

- Clearly state that the work is simulation-based.

- Use British English spelling where appropriate.

- Avoid using em dashes.