| Sensor Type | Model | Manufacturer | Core Parameters (Matching Paper Requirements) | Functions | Model Description | Product Source (Official Website/Industry Case) |
| --- | --- | --- | --- | --- | --- | --- |
| Electrode Force Sensor | HBM U10M-5kN | HBM (Germany, Industrial-grade) | Range: 0-5kN (covers 1500-3500N in Paper 1), Accuracy: ±0.1% (exceeds ±1% required in the paper), Sampling Rate: 1kHz, Temperature Resistance: -30~+80℃ | Real-time monitoring of electrode pressure and feedback for parameter adjustment (Paper 1, Table 2: Electrode force contributes 70.95% to weld strength) | Industrial Adaptive Selection (No model specified in the paper) | HBM Official Website - U10M Series |
| Welding Current Sensor | LEM LA 55-P | LEM (Switzerland, Automotive-grade) | Range: 0-50A (compatible with welding current 3-9kA, requires shunt), Accuracy: 0.5% (0.5% required in Paper 1), Output: 0-5V analog signal | Collect welding current fluctuations; pause welding when fluctuation exceeds ±5% (Paper 1, Table 3: Current contributes 23.35% to weld strength) | Industrial Adaptive Selection (No model specified in the paper) | LEM Official Website - LA Series |
| Ultrasonic Inspection Module | VOGT PHAsisBLU | VOGT Ultrasonics (Germany) | 20MHz matrix probe (11×11=121 elements), 729 measurements/weld, D-scan Resolution: 0.35mm, Almost Couplant-Free Inspection | Identify nugget area (≥50mm² as qualified), cold welds, and pores (Paper 6, Pages 13-14) | Paper-Explicit Model (Empirical Product) | VOGT Official Website - PHAsis Series |
| Industrial Camera | Keyence IV2 Series (IV2600) | Keyence (Japan) | Resolution: 1280×1024, Positioning Accuracy: ±0.01mm (exceeds ±15mm in Paper 6), Frame Rate: 30fps, Temperature Resistance: 0~+50℃ | Locate welds (correct posture deviation) and detect spatter (grayscale threshold method) (Paper 6, Page 19) | Industrial Adaptive Selection (No model specified in the paper) | Keyence Official Website - IV2 Series |
| Electrode Tip Temperature Sensor | Omron E52-CA1D | Omron (Japan) | Range: 0-300℃ (covers 250℃ replacement threshold in Paper 1), Accuracy: ±0.5℃, Non-contact Infrared, Response Time: 100ms | Monitor electrode tip temperature; prompt replacement when >250℃ (avoid adhesion) (Paper 1, Section 4.3) | Industrial Adaptive Selection (No model specified in the paper) | Omron Official Website - E52 Series |
| Robot Encoder | ABB IRC5 Integrated Encoder (3HAC035609-001) | ABB Robotics | Resolution: 16-bit, Sampling Rate: 1kHz, Pulses per Revolution: 1024, Vibration Resistance: 1000m/s² | Record joint angles to ensure spot welding posture repeatability of ±0.02mm (Paper 2: Input for FNN trajectory optimization) | Paper-Explicit Model (Robot Native Component) | ABB Official Website - IRC5 Spare Parts |
| Grinding-Spraying Integrated Device | VOGT Custom Model (PHAsis-Matched) | VOGT Ultrasonics | Grinding Head Service Life: 5,000 cycles, Couplant Capacity: 10L (for 200,000 welds), Single Spray Volume: 0.035ml | Grind spatter and spray micro-couplant before inspection (Parameters in Paper 6, Page 21) | Paper-Explicit Model (Matched Product) | Empirical Description in Paper 6, Page 21 (Co-developed by VOGT and ABB) |
| Vibration Sensor (Joint Monitoring) | Kistler 8763B | Kistler (Switzerland) | Range: ±50g, Sampling Rate: 1kHz, Accuracy: ±0.5%, Temperature Resistance: -55~+125℃, Suitable for robot joint installation | Monitor vibrations of Joints 2/3 and extract RMS values to warn of reducer wear (Paper 3, Figure 4.19) | Industrial Adaptive Selection (Supplementary joint monitoring requirement) | Kistler Official Website - 8763B Series |

| Agent Name | Core Functionality | Algorithms/Methods Used |
| --- | --- | --- |
| Data Agent | 1Data acquisition, 2parsing, cleaning, feature engineering, and distribution. | - Linear interpolation (for missing value imputation) - Z-Score standardization (for data normalization) - Grayscale threshold method (for spatter detection from camera data) |
| Predictive Agent | Weld quality prediction, optimal parameter recommendation, and electrode life prediction. | - XGBoost Regression (for weld tensile shear strength prediction) - Taguchi Method (for optimal spot welding parameter selection) - Linear Regression (for electrode tip life prediction) |
| Decision agent | Take a decision |  |
| Collaboration Agent | Full-process weld control, equipment maintenance linkage, and integration with factory systems (MES/ERP). | - Rule-based decision making (for weld quality handling: qualified/unqualified/unrepairable) - Real-time system integration protocols (for MES/ERP interaction) |
| Human-Agent Interaction Agent | Visualization, quality traceability, manual operation, and feedback to other agents. | - Data visualization techniques (for real-time dashboard and trend reports) - User input handling and feedback integration (for model iteration and alarm management) |

Reinforcement Learning from Human Feedback (RLHF) aligns AI with human preferences via 3 key steps:

1. Supervised Fine-Tuning (SFT): Train the model on human-labeled "task-response" pairs to form an initial strategy.

2. Reward Model (RM) Training: Humans rank the model’s multiple outputs for the same task; the model learns to score human preferences.

3. Policy Optimization (e.g., PPO): The model updates its behavior using RM scores as rewards, avoiding big deviations from human expectations.