Evaluation of Machine Learning Models for Predictive Maintenance in Industrial Spot Welding

1. Introduction

To identify the optimal predictive maintenance model for an industrial spot welding robot. This report evaluated 18 machine learning algorithms on its operational data. The primary objective is to achieve accurate and timely prediction of equipment maintenance needs, thereby providing data-driven decision support for smart manufacturing initiatives.

To accomplish this, a robust and reproducible automated evaluation framework was developed to conduct a comprehensive comparison of multiple candidate models across key dimensions, including predictive performance, stability, and computational cost.

2. Experimental Design

A rigorous experimental protocol was designed to ensure the scientific validity and integrity of the evaluation results.

The study utilizes the spot_welding_dataset_100k_adjusted.csv dataset, which contains 100,000 records of spot welding operations. Six key physical parameters were selected as input features for the models:

Electrode_Force_N, Welding_Current_kA, Ultrasonic_Inspection_Score, Electrode_Tip_Temp_C, Robot_Encoder_Error_mm, and Vibration_Sensor_RMS_ms2.

Prior to model training, the feature data underwent Standard Scaling to normalize the ranges of different physical units. This preprocessing step ensures that all models learn on a fair and equitable basis. The target variable, representing maintenance categories, was numerically encoded using label encoding.

To facilitate a broad and thorough comparison, we curated a library of 18 distinct model configurations representing seven major families of machine learning algorithms. This diverse selection was chosen to comprehensively explore the potential of different modeling paradigms in addressing this specific problem. The candidate library includes:

Baseline Models: Logistic Regression, Gaussian Naive Bayes

Instance-Based Models: K-Nearest Neighbors (KNN)

Tree Models: Single Decision Tree

Kernel Methods: Support Vector Machine (SVM)

Ensemble Learning - Bagging: Random Forest

Ensemble Learning - Boosting: Scikit-learn Gradient Boosting, XGBoost, LightGBM

Neural Networks: Multi-layer Perceptron (MLP)

The core evaluation protocol employed was 5-Fold Stratified Cross-Validation. This method partitions the dataset into five subsets, iteratively using four for training and one for validation, repeating the process five times. This ensures that every data point is used for both training and validation, leading to a more robust estimate of the model's generalization performance on unseen data.

To further assess model stability, the entire 5-fold cross-validation process was independently repeated five times, each with a different random seed. The final performance metrics are reported as the mean and standard deviation of the results from these five independent runs.

*All experiments were conducted with PyCharm2024.3.6 on a HUAWEI MDG-XX workstation with the following specifications:

CPU: 13th Gen Intel(R) Core(TM) i5-13420H (~2.1GHz, 12 CPUs)

RAM: 16 GB

Operating System: Windows 11 Home 64-bit

3. Results and Trade-off Analysis.

The automated evaluation pipeline produced a comprehensive performance summary for all 18 model configurations. The results, aggregated across all five runs, are presented below.

Comprehensive Performance table:

This evaluation demonstrates that top-tier models, including Multi-layer Perceptrons and Boosting ensembles, converge on a near-identical peak accuracy of approximately 93.6%. The primary differentiator is not performance but computational efficiency. The XGBoost (Fast) configuration achieved 93.62% accuracy in just 5.42 seconds, nearly five times faster than the top-performing MLP (93.69% in 26.91 seconds) for a statistically insignificant performance difference. Simpler models like Logistic Regression (~85%) and Naive Bayes (67%) were clearly inferior. XGBoost (Fast) is therefore the optimal choice, offering the best trade-off between accuracy and efficiency.

4. Analysis of Algorithm Performance and Causality

A detailed analysis of each algorithm's performance provides insight into the nature of the dataset and the suitability of different modeling approaches.

1.Multi-layer Perceptron (MLP):

The MLP (Simple Net) achieved the highest mean accuracy (93.69%). Its success stems from its ability to model complex, non-linear relationships through its layered structure of neurons and activation functions. This indicates that the relationship between the sensor readings and maintenance needs is not simple or linear, requiring a model that can learn hierarchical and abstract feature representations.

2.Boosting Ensembles (XGBoost, LightGBM, Gradient Boosting):

This family of algorithms was the most consistent high-performer. Their strength lies in their sequential, error-correcting nature; each tree in the ensemble is built to correct the mistakes of the previous ones. This iterative refinement allows them to fit the data very precisely. The remarkable speed of XGBoost (Fast) (5.42s) and LightGBM (Fast) (7.44s) compared to Scikit-learn's Gradient Boosting (282.65s) highlights the advanced engineering and optimization of these modern libraries.

3.Random Forest:

As a bagging ensemble, Random Forest performed robustly (93.58%). It operates by constructing many independent decision trees on different subsets of the data and averaging their predictions. This "wisdom of the crowd" approach makes it highly resistant to overfitting and very stable, as evidenced by its low standard deviation.

4.Decision Tree:

The pruned decision tree (93.20%) significantly outperformed the unconstrained, overfit version (91.63%). This is a classic demonstration of the bias-variance trade-off. The overfit tree memorized the training data too closely, including its noise, while the pruned tree learned simpler, more generalizable rules, leading to better performance on unseen validation data.

5.Support Vector Machine (SVM):

On the 10k-sample subset, the RBF kernel SVM (91.19%) outperformed the linear kernel (85.72%). This result confirms the non-linear nature of the problem. The RBF kernel's ability to map features to a higher-dimensional space allows it to find a non-linear separating boundary, which is more effective here than a simple linear one.

6.K-Nearest Neighbors (KNN):

The performance of KNN was moderate. This algorithm classifies a point based on the majority class of its nearest neighbors. Its effectiveness is highly depe.ndent on the feature space having a meaningful distance metric. While better than linear models, its performance suggests that simple proximity is not as powerful a predictor as the hierarchical rules learned by tree ensembles.

7.Logistic Regression:

This model provided a strong linear baseline at around 85.12%. Its performance indicates that there is a discernible linear component to the problem, but its inability to compete with the top models confirms that the decision boundaries are fundamentally non-linear.

8.Gaussian Naive Bayes:

This was the worst-performing model by a significant margin (67.03%). Its core assumption is that all features are conditionally independent of one another. Its failure strongly implies that this assumption is violated in the dataset—for example, it is highly likely that electrode temperature and vibration levels are correlated.

5. Conclusion

Based on a comprehensive evaluation across 18 model configurations, the key conclusion is that while multiple advanced models achieve top-tier accuracy, the XGBoost (Fast) configuration stands out as the definitively optimal choice for this predictive maintenance task.

Codes：

import pandas as pd
import time
import warnings
import numpy as np
from sklearn.preprocessing import StandardScaler, LabelEncoder
from sklearn.model_selection import cross_val_score, StratifiedKFold, train_test_split
from sklearn.ensemble import RandomForestClassifier, GradientBoostingClassifier
from sklearn.svm import SVC
from sklearn.linear_model import LogisticRegression
from sklearn.neighbors import KNeighborsClassifier
from sklearn.tree import DecisionTreeClassifier
from sklearn.naive_bayes import GaussianNB
from sklearn.neural_network import MLPClassifier
import xgboost as xgb
import lightgbm as lgb
from tqdm import tqdm

# --- SETUP: Suppress warnings for a clean output ---
from sklearn.exceptions import ConvergenceWarning

warnings.filterwarnings("ignore", category=UserWarning)
warnings.filterwarnings("ignore", category=ConvergenceWarning)

# --- 1. Data Loading and Preparation ---
print("--- Loading and Preparing Data for Comprehensive Evaluation ---")
# ... (Data loading remains the same) ...
file_path = 'spot_welding_dataset_100k_adjusted.csv'
try:
    df = pd.read_csv(file_path)
except FileNotFoundError:
    print(f"Error: Dataset '{file_path}' not found.")
    exit()
features = ['Electrode_Force_N', 'Welding_Current_kA', 'Ultrasonic_Inspection_Score', 'Electrode_Tip_Temp_C',
            'Robot_Encoder_Error_mm', 'Vibration_Sensor_RMS_ms2']
X = df[features]
scaler = StandardScaler()
X_scaled = scaler.fit_transform(X)
Y_maintenance = df['Predictive_Maintenance_Category']
le_maintenance = LabelEncoder()
Y_encoded = le_maintenance.fit_transform(Y_maintenance)


# --- 2. Function to Generate Experiments for a GIVEN SEED ---
def get_experiments(seed):
    # The experiment list remains the same
    experiments = [
        {"family": "1. Baseline Models", "name": "Logistic Regression (Low Reg)",
         "model": LogisticRegression(C=10, max_iter=1000, random_state=seed)},
        {"family": "1. Baseline Models", "name": "Logistic Regression (High Reg)",
         "model": LogisticRegression(C=0.1, max_iter=1000, random_state=seed)},
        {"family": "1. Baseline Models", "name": "Gaussian Naive Bayes", "model": GaussianNB()},
        {"family": "2. Instance-Based", "name": "KNN (K=3, Sensitive)",
         "model": KNeighborsClassifier(n_neighbors=3, n_jobs=-1)},
        {"family": "2. Instance-Based", "name": "KNN (K=15, Smooth)",
         "model": KNeighborsClassifier(n_neighbors=15, n_jobs=-1)},
        {"family": "3. Tree Models", "name": "Decision Tree (Overfit)",
         "model": DecisionTreeClassifier(random_state=seed)},
        {"family": "3. Tree Models", "name": "Decision Tree (Pruned)",
         "model": DecisionTreeClassifier(max_depth=10, min_samples_leaf=20, random_state=seed)},
        {"family": "4. Kernel Methods", "name": "SVM (Linear Kernel)",
         "model": SVC(kernel='linear', probability=True, random_state=seed), "use_subset": True},
        {"family": "4. Kernel Methods", "name": "SVM (RBF Kernel, C=1)",
         "model": SVC(kernel='rbf', C=1, probability=True, random_state=seed), "use_subset": True},
        {"family": "5. Bagging Ensemble", "name": "Random Forest (Fast)",
         "model": RandomForestClassifier(n_estimators=50, max_depth=10, random_state=seed, n_jobs=-1)},
        {"family": "5. Bagging Ensemble", "name": "Random Forest (Accurate)",
         "model": RandomForestClassifier(n_estimators=200, max_depth=20, min_samples_leaf=5, random_state=seed,
                                         n_jobs=-1)},
        {"family": "6. Boosting Ensemble", "name": "Gradient Boosting (Scikit)",
         "model": GradientBoostingClassifier(n_estimators=200, learning_rate=0.1, max_depth=5, random_state=seed)},
        {"family": "6. Boosting Ensemble", "name": "XGBoost (Fast)",
         "model": xgb.XGBClassifier(n_estimators=150, max_depth=6, learning_rate=0.1, objective='multi:softmax',
                                    num_class=len(le_maintenance.classes_), use_label_encoder=False,
                                    eval_metric='mlogloss', random_state=seed, n_jobs=-1)},
        {"family": "6. Boosting Ensemble", "name": "XGBoost (Accurate)",
         "model": xgb.XGBClassifier(n_estimators=300, max_depth=8, learning_rate=0.05, objective='multi:softmax',
                                    num_class=len(le_maintenance.classes_), use_label_encoder=False,
                                    eval_metric='mlogloss', random_state=seed, n_jobs=-1)},
        {"family": "6. Boosting Ensemble", "name": "LightGBM (Fast)",
         "model": lgb.LGBMClassifier(n_estimators=150, learning_rate=0.1, objective='multiclass',
                                     num_class=len(le_maintenance.classes_), random_state=seed, n_jobs=-1)},
        {"family": "6. Boosting Ensemble", "name": "LightGBM (Accurate)",
         "model": lgb.LGBMClassifier(n_estimators=300, learning_rate=0.05, num_leaves=40, objective='multiclass',
                                     num_class=len(le_maintenance.classes_), random_state=seed, n_jobs=-1)},
        {"family": "7. Neural Network", "name": "MLP (Simple Net)",
         "model": MLPClassifier(hidden_layer_sizes=(50,), max_iter=500, random_state=seed)},
        {"family": "7. Neural Network", "name": "MLP (Deeper Net)",
         "model": MLPClassifier(hidden_layer_sizes=(100, 50), max_iter=500, random_state=seed, early_stopping=True)}
    ]
    return experiments


# --- 3. Run Comprehensive Evaluation Across Multiple Seeds ---
all_results = []
num_runs = 5  # Set the number of runs for stability testing
subset_size = 10000

for i in range(num_runs):
    run_seed = i * 42
    print(f"\n--- Starting Run {i + 1}/{num_runs} with random_state={run_seed} ---\n")

    experiments = get_experiments(run_seed)
    cv_strategy = StratifiedKFold(n_splits=5, shuffle=True, random_state=run_seed)

    X_subset, _, Y_subset, _ = train_test_split(
        X_scaled, Y_encoded, train_size=subset_size,
        stratify=Y_encoded, random_state=run_seed
    )

    for experiment in tqdm(experiments, desc=f"Run {i + 1} Evaluation"):
        model_name = experiment["name"]
        model = experiment["model"]

        if experiment.get("use_subset", False):
            X_eval, Y_eval = X_subset, Y_subset
            eval_name = model_name + f" (on {subset_size // 1000}k subset)"
        else:
            X_eval, Y_eval = X_scaled, Y_encoded
            eval_name = model_name

        start_time = time.time()
        scores = cross_val_score(model, X_eval, Y_eval, cv=cv_strategy, n_jobs=-1, scoring='accuracy')
        total_time = time.time() - start_time

        all_results.append({
            "Model Configuration": eval_name,
            "Accuracy": scores.mean(),
            "Time (s)": total_time
        })

# --- 4. Aggregate and Display the Final Comprehensive Report ---
print("\n" + "=" * 80)
print("--- FINAL COMPREHENSIVE REPORT: Model Performance, Stability, and Cost ---")
print(f"--- (Aggregated from {num_runs} full runs of 5-fold cross-validation) ---")
print("=" * 80)

results_df = pd.DataFrame(all_results)

# Use groupby and agg to calculate mean and std for BOTH accuracy and time
final_report = results_df.groupby('Model Configuration').agg(
    Mean_Accuracy=('Accuracy', 'mean'),
    Std_Dev_Accuracy=('Accuracy', 'std'),
    Mean_Time_s=('Time (s)', 'mean'),
    Std_Dev_Time_s=('Time (s)', 'std')
).reset_index()

# Format the final table for presentation
final_report['Mean Accuracy'] = final_report['Mean_Accuracy'].map('{:.2%}'.format)
final_report['Accuracy Stability (Std Dev)'] = final_report['Std_Dev_Accuracy'].map('{:.4f}'.format)
final_report['Mean CV Time (s)'] = final_report['Mean_Time_s'].map('{:.2f}'.format)

# Sort by the most important metric: Mean Accuracy
final_report = final_report.sort_values(by='Mean_Accuracy', ascending=False)

# Select and rename columns for the final report
report_columns = [
    'Model Configuration',
    'Mean Accuracy',
    'Accuracy Stability (Std Dev)',
    'Mean CV Time (s)'
]
print(final_report[report_columns].to_markdown(index=False))

Comments for improvements

Data Visualization

Convert the Results Table (Section 3) into a visual plot.	A scatter plot of Mean Accuracy (Y-axis) vs. Mean CV Time (X-axis) would visually isolate the top-performing models (MLP, Boosting) and immediately highlight the XGBoost (Fast) configuration as the superior trade-off point. This single visual will make the core finding instantly clear to any reader.

Additional Metrics

Include a table of Precision, Recall, and F1-Score for the top 5 models.	Accuracy is effective but can be misleading in classification problems, especially if the maintenance categories are imbalanced. For predictive maintenance, Recall (ensuring all failure cases are caught) or Precision (avoiding costly false alarms) is often more important. The F1-Score is a better holistic measure.

Feature Importance

Add a section detailing Feature Importance. Use the optimal model (XGBoost) or a robust ensemble (Random Forest) to calculate and visualize the importance of the 6 input features. This analysis answers why the model works, identifying which physical parameters are the strongest predictors of maintenance needs (e.g., Is Vibration_Sensor_RMS_ms2 or Electrode_Force_N most critical?).

Deployment Feasibility

Discuss the XGBoost model's suitability for edge or embedded deployment.	The model is fast (5.42s mean CV time), making it highly suitable for real-time operation. Mention that its small memory footprint (compared to large neural networks) makes it ideal for deployment directly on a supervisory industrial IoT gateway (edge device), a key challenge in smart manufacturing.